TO CONNECT HK2 TO KEYWATCHER III

	CONNECTOR

TYPE
	KW CPU BOARD

(counting from right to left)
	HK2 Hand Reader

(Old Style F1 Board)
	HK2 Hand Reader

(New Style F3 Board)

	Data 0
	Connector 3 (white)
	Connector 21
	Connector 6

	Data 1
	Connector 4 (green)
	Connector 19
	Connector 5

	Ground
	Connector 6 (black)
	Connector 20
	Connector 4

	Ground Strapping
	Not connected
	Connector 20
	Connector 4


EXAMPLE KEYWATCHER 3 TO F3 SERIES HAND READER

[image: image1.jpg]Notes: Key watcher and Hand Reader must both
F3 Series Hand Reader Side be powered down before attempting any connections.

Pin 4= Black (Ground) And Ground Strapping  is hooked to 1 of the 4 nand reader
Pin 4= Ground Strapping back plate screws.
Pin 5= Green (Data 1)
Pin 6= White (Data 0)

Note: Red wire NOT used!

(belden 82723
or 87723
shielded 2
twisted pair)

Key Watcher Side

Pin 3= White (Data 0)
Pin 4= Green (Data 1)
Pin 6= Black (Ground)

Note: Ground Strapping and
Red wire NOT used!





